I do not have any documentation for the game at this time but I will try to

inform you of some important aspects of my game, and how to set it up.

USER SPECIFIC FILES

What files can be made available to users:

TOWERTXT.ZIP contains example text files to illustrate how to make

adventures, to add for other users to play.  It is suggested that you test

these files before adding them to your game.  There is no test EXE file at

present.

*** NOTE *** No other files can be made available to anyone from your BBS.

ADVENTURE FILES

Multiple files may be located in the game directory of these three types:

*.MON : Contains monster definitions.

*.ADF : Contains room and command definitions.

*.ADT : Contains object definitions.

All monsters, and objects are loaded when the game is started.  Only *.ADF

files are read when starting the game or switching between other *.ADF files.

This file name can not be changed:

MAINSYN.TXT : Contains all synonyms for input text.

GAME FILES

TOWER.EXE 	: Main program.

TOWER.OVR 	: Overlay file.

MAINT.EXE 	: Maintenance program.

EDITOR.EXE      : Configuration program.

GAME.CTL 	: Game control file.  Contains system information including

non-standard communications ports.

HARDWARE REQUIREMENTS

386 or better processor : REQUIRED

Expanded Memory : recommended

250K memory for the game itself : required

200K or more for game information : recommended

386 or better processor is REQUIRED.  Later versions will work with a 286.

Expanded Memory is recommended to load the overlay into.  The

overlay will be read from disk if EMS is not present, but the game will not

operate as quickly.  450K is recommended to run this game.  It is a memory

HOG and will run with less memory, however if memory is not available to the

program for vital functions the game will return the user to the BBS without

notification.  This program is not yet optimized for low memory systems, but

should work just fine.  This memory requirement is in the standard 640K block

for running programs.  The program does not use DPMI at present.

HOW TO SET UP TOWER

To set the game up, unzip the contents of the TOWERXXX.ZIP into a directory

you create.  If you unzipped this into a temporary directory, move all the

files contained in the TOWERXXX.ZIP to a directory of your choice.

Run EDITOR.EXE to configure how many nodes you have.  Do not include the

local node.  A local node is assumed.  Local mode is also forced to be

node #0.  So if you only have one line accessing your BBS you have 1 node.

Here is an example command line for node one on any BBS:

TOWER /N1 /PD:\BBS\NODE1\  ; /Nx is the node number.

; /Pxxx is the path to the BBS drop file

; Do not indicate the drop file.

WHAT ARE DROP FILES?

When the BBS transfers control to a door program it provides a file, called a

"DROP FILE" to the door program which contains essential information such as

the users name, time remaining, com port, connect speed, etc..  The format

for this "DROP FILE" varies from BBS to BBS.  At this time the

Tower Of Shadows Package supports the following drop file formats:

DOORSYS - The DOOR.SYS format (Wildcat!,GAP,TAG, etc)

*DORINFO1 - Remote Access, others(uses DORINFO1.DEF**)

RBBS - Rbbs version 16.1+  (uses DORINFOx.DEF*)

QUICK - Quickbbs, Force , R.A. (uses DORINFOx.DEF**)

PCB14 - Pcboard version 14

PCB15 - Pcboard version 15

PHOENIX - Phoenix BBS (uses INFO.BBS)

SPITFIRE - Spitfires BBS (uses SFDOORS.DAT)

WWIV - WWIV BBS (uses CHAIN.TXT)

TRIBBS - TriBBS software (uses TRIBBS.SYS)

2AM - JUMPER.DAT

* New BBS type DORINFO1 uses the dorinfo1.def for dropfile name.

PCBoard 12 and Callinfo support herewith ends. These two obsolete dropfiles

are no longer supported.

THE DOOR CONTROL FILE (CTL)

To run properly Tower Of Shadows requires configuration information from a

control (CTL) file.  For your use a GAME.CTL file is enclosed with this

package.  The GAME.CTL file is unedited and heavily commented so that you

might understand what each line does.  Note that lines beginning with the

";" character are NOT read by the Tower Of Shadows program.  This CTL file

tells Tower Of Shadows how to act, what drop file to use, etc.  It should

be noted that Tower Of Shadows will NOT function without the presence of

a CTL file.  The CTL file can be renamed, such as GOC.CTL or RAW.CTL and

is loaded and processed automatically when your BBS runs Tower Of Shadows.

It has been my experience that most problems experienced by users of

Tower Of Shadows are from improperly configured CTL files.  To help correct

this Steven Lorenz has created a standard CTL setup program which is

distributed with Tower Of Shadows.  This setup program (DDSETUP.ZIP)

provides VGA and mouse support and is very professional in appearance.

Command Line Parameters:

Tower Of Shadows supports several parameters which may be specified on the

command line. These parameters are handled automatically when you run the

program.  An example would be if you wanted to run the GodFather of Crime

Door Game locally then you would type the Command Line parameter:  GOC /L

The following is a list of Tower Of Shadows supported Command Line Parameters.

"/Bxxxxx"   This tells GAME.EXE to lock its baud rate at xxxxx.  In order to

get the most efficiency out of a high speed error correcting

modem (9600 baud or above), you may lock the baud rate to a speed

higher than what the caller is actually using.  By locking the

baud to say 38,400, the modem can compress text data that is

being sent and achieve a baud rate much higher than the actual

transmission rate.  This will lock the baud rate at the same

value for all nodes that use this command line.  If you prefer

a more selective way to lock baud use 'LOCKBAUD' command in the

GAME.CTL file.

"/Cx"       (where x is 1..4) This specifies a comport number.  Some BBS

software does not create door information files (dropfiles) that

contain the comport number.  If your BBS does have a comport

listed in the dropfile then there is no need for this parameter.

The default is comport 0, local mode.

"/L"        This tells GAME.EXE to load up in LOCAL mode.  This is good for

testing or when a sysop just wants to "try out" the door without

messing with his BBS setup.  The "/L" mode will ask the user for

his name and run the door in local mode.

"/Nx"       (where x is 1..9) This specifies a "node number".  Some BBS

software support multinode environments in which you must know

the node number of the drop file to use. For example, RBBS-PC

uses DORINFOx.DEF. If you specified "/N2" then GAME.EXE would

use DORINFO2.DEF.

"/Mx"       (where x is up to 2 filename acceptable characters) Used to

specify one of Multiple control files.  If you have created more

than one GAME.CTL file (GAME1.CTL, GAME2.CTL, etc.) for each node

or network PC then you can select the one you want by using the

/M parm.  Example: "/M2" will force GAME.EXE to use GAME2.ctl.

"/Pyyy"     This specifies the path to the drop files.  For example,

"/PC:\WILDCAT\WCWORK\NODE1" would tell GAME.EXE to look for the

drop file in C:\WILDCAT\WCWORK\NODE1. If this is left out the

program assumes the drop file is local.

"/V"        This command will tell GAME.EXE not to show any display on the

local screen.  Only a colored screen and a message will be

displayed.  The cursor will not move as it will be turned off.

Saves a little processing on multi-tasking systems.

"/W"        This flags the program that a network is present even though no

multitaskers are detected.  This is useful if the door is run on

a network such as Novell that doesn't use multitaskers.

Special Note on Multi-Tasker Support

Tower Of Shadows can detect the presence of DesqView, OS2, Windows 3.1, and

Windows NT and provide automatic time slice releasing to them.

NOTE **** If using OS/2 disable all DesqView task masking, or fake time

slice releasing that fools programs without direct OS/2 support into thinking

they are running under DesqView.

Need more Help

Contact me at

danielh@teleport.com

Daniel Holmes 1:105/98

Or on one of these fidonet echos:

BBSDOORS      Door Games relating to all bulletin board systems

BBS_DOORS     Discussions of all types of BBS Doors and their Distrib

DOORGAMES     BBS Doorgames and discussions

DOORWARE      General Door Development Forum

ON_LINE_GAMES Talk about your Favorite ON LINE GAME